Code Arduino ide

#include <Adafruit_MPU6050.h>

#include <Adafruit_Sensor.h>

#include <WebSocketsClient.h> // เปลี่ยนมาใช้ Library ของ Links2004

#include <WiFi.h>

#include <Wire.h>

#include <time.h>

// ================= WIFI & CLOUD =================

const char *ssid = "wifi name";

const char *password = "password your wifi";

const char *wsHost = "smart-sandbag-server.onrender.com";

const int wsPort = 443;

// ================= OBJECT =================

WebSocketsClient webSocket;

Adafruit_MPU6050 mpu;

// ================= STATE =================

int rep = 0;

bool running = false;

bool aboveUp = false;

bool wsConnected = false;

unsigned long lastMsgTime = 0;

// ================= ANGLE =================

float angleOffset = 0;

float smoothAngle = 0;

const float ALPHA = 0.15;

float UP_ANGLE = 45;

float DOWN_ANGLE = 30;

// ================= FUNCTION PROTOTYPES =================

float readRawAngle();

float calibrateAngle();

void syncTime();

void sendDeviceStatus(String statusText);

void webSocketEvent(WStype_t type, uint8_t *payload, size_t length);

// ================= UTIL FUNCTIONS =================

float readRawAngle() {

sensors_event_t a, g, t;

mpu.getEvent(&a, &g, &t);

return atan2(a.acceleration.x, a.acceleration.z) * 180 / PI;

}

float calibrateAngle() {

float sum = 0;

for (int i = 0; i < 50; i++) {

sum += readRawAngle();

delay(10);

}

return sum / 50;

}

void syncTime() {

configTime(7 * 3600, 0, "pool.ntp.org", "time.nist.gov");

Serial.print("⏳ NTP Syncing");

while (time(nullptr) < 1000000000l) {

delay(500);

Serial.print(".");

}

Serial.println("\n✅ Time Synced!");

}

void sendDeviceStatus(String statusText) {

if (wsConnected) {

String json =

"{\"type\":\"status\",\"deviceStatus\":\"" + statusText + "\"}";

webSocket.sendTXT(json);

}

}

// ================= WEBSOCKET EVENT HANDLER =================

void webSocketEvent(WStype_t type, uint8_t *payload, size_t length) {

switch (type) {

case WStype_DISCONNECTED:

Serial.println("❎ WebSocket Disconnected!");

wsConnected = false;

break;

case WStype_CONNECTED:

Serial.println("🔌 WebSocket Connected!");

wsConnected = true;

sendDeviceStatus("อุปกรณ์พร้อมใช้งาน (Online)");

break;

case WStype_TEXT: {

String msg = (char *)payload;

if (msg.indexOf("\"type\":\"sensor\"") >= 0)

return;

if (msg.indexOf("reset") >= 0) {

sendDeviceStatus("กำลังรีบูต...");

delay(500);

ESP.restart();

}

if (msg.indexOf("running") >= 0) {

if (msg.indexOf("true") >= 0) {

running = true;

sendDeviceStatus("กำลังทำงาน (Running)");

} else {

running = false;

sendDeviceStatus("หยุดชั่วคราว (Paused)");

}

}

break;

}

case WStype_ERROR:

Serial.println("❌ WS Error!");

break;

}

}

// ================= SETUP =================

void setup() {

Serial.begin(115200);

delay(1000);

Wire.begin(21, 22);

WiFi.begin(ssid, password);

while (WiFi.status() != WL_CONNECTED) {

delay(500);

Serial.print(".");

}

Serial.println("\n✅ WiFi Attached");

syncTime();

if (!mpu.begin()) {

Serial.println("❌ MPU6050 error");

while (1)

;

}

Serial.println("🧘 Calibrating...");

angleOffset = calibrateAngle();

Serial.println("✅ Calibrated");

// --- WebSocket Setup ---

webSocket.beginSSL(wsHost, wsPort, "/"); // เชื่อมต่อผ่าน WSS

webSocket.onEvent(webSocketEvent);

webSocket.setReconnectInterval(5000); // รองรับการต่อใหม่กรณีหลุด

// เพิ่ม Header สำคัญสำหรับ Cloud

webSocket.setExtraHeaders(

"Origin: https://smart-sandbag-server.onrender.com");

}

// ================= LOOP =================

void loop() {

webSocket.loop();

float raw = readRawAngle() - angleOffset;

smoothAngle = ALPHA * raw + (1 - ALPHA) * smoothAngle;

if (running) {

if (!aboveUp && smoothAngle > UP_ANGLE)

aboveUp = true;

if (aboveUp && smoothAngle < DOWN_ANGLE) {

rep++;

aboveUp = false;

}

}

// ส่งข้อมูลทุกๆ 100ms ถ้าเชื่อมต่ออยู่

if (wsConnected && (millis() - lastMsgTime > 100)) {

lastMsgTime = millis();

String msg = String("{\"type\":\"sensor\",\"payload\":{") +

"\"angle\":" + String(smoothAngle, 2) + "," +

"\"rep\":" + rep + "," +

"\"running\":" + (running ? "true" : "false") + "," +

"\"deviceStatus\":\"Online\"" + "}}";

webSocket.sendTXT(msg);

}

}